3D points, spheres, etc. (for a femur; the femoral neckshaft

angle (FNSA), the femoral head and posterior condyles

representing spheres) and a full 3D morpho-realistic

personalized parametric model (MPPM) of the lower limb

obtained by correcting the SPPM. The Kriging methodology

was used for global deformation and local adaptation.

The as-rigid-as-possible deformation method was used

for local adaptation which is based on the moving leastsquares

approach. Then 3D image reconstruction method

was applied in two steps; a fast 3D lower limb reconstruction

and a full 3D lower limb reconstruction. This was

the first 3D modeling approach for the whole lower limb

using biplanar X-rays with FT (femoral torsion) and TT

(Angle between Tibial Plates Axis and the Bimalleolar

Axis) calculations.

Cresson et al. [19] tried to overcome the drawback of

overlapping regions to infer the information of location and

shape of the hidden portions of the spine. The algorithm

operates using two iterating processes. The first process uses

a sophisticated 2D/3D registration procedure. Boundary

edges were retrieved from radiographs to create a customized

model of a vertebra. The second process refines the

reconstruction of all quasi sections of the spine by registering

estimates of anatomical features using a statistical

model. The suggested technique can be a dependable alternative

when compared with state-of-the-art technologies.

Convolution Neural Network (CNN) is an effective solution

in a variety of applications viz; image processing [20,

21], image segmentation [22, 23], pattern recognition [24,

25] and computer vision [20]. The author Kasten et al. [26]

addressed the issues of an absence of heuristic knowledge

and dimensional expansion with conventional differential

layers. It employed a dimensional augmentation technique

where each pair of matching epipolar lines was back-projected

into a two-channeled epipolar plane by bi-planar

X-rays. A 3D structure was produced by combining an augmentation

technique with a deep learning architecture. This

method to generate 3D representations of the various bones

maintains the geometric limits of the two views. To achieve

a more robust, accurate and efficient result, domain adaptation

is used.

Dixit et al. [27] used a machine learning approach for the

construction of 3D models from 2D X-ray images. Features

extracted are the color, size and depth of the femur bone.

Depth information was used to create a mesh point cloud.

The image was then converted into STL (Stereolithography)

representation. A 3D model was created using CNN.

From X-ray projection images obtained in an upright

position, the author Akkoul et al. [28] created a 3D customized

model of the femur. The strategy was based on a

two-stage pseudo-stereo matching process. The coordinates

of a 3D contour are determined in the first stage using two

sequential projections. The contours of the proximal femur

are derived from the 2D X-ray images. The city-block, the

Euclidian 2D spatial distances and the Chessboard were

compared to match the points of two contours. Then, point

pairs estimated are used to build a collection of threedimensional

points. The surface model was reconstructed

using a meshing approach from the cloud of points based

on Poisson's equation, resulting in a closed 3D surface. To

compensate for the absence of information and increase the

quality of the rebuilt 3D surface, a 3D reference surface of

the proximal femur was employed in the second stage to

inject extracted points locally into the reconstructed surface.

Thus, in this technique of merging numerous 3D points created

from a reference model obtained from a CT scan, the

use of an X-ray stereo model in conjunction with a shape

constraint improves accuracy and reduces error.

The statistical shape model (SSM) is a mathematical

model incorporating information on the shape as well as its

variance. Lamecker et al. [40] used a 3D Statistical shape

Model to reconstruct the 3D shape from a few digital X-ray

images. The author extracted the silhouettes from the projection

using statistical shape training model thickness and

the simulated X-rays. The approach optimizes a similarity

measure analyzing the difference between the projections of

the shape model and X-ray images by measuring the distance

between the object silhouettes in the projections.

Zhu et al. [32] constructed 3D geometric surface models

of the human knee joint. This work developed an enhanced

SSM approach for predicting the 3D joint surface model that

only utilizes 2D images of the joint. A total of 40 human

knee distal femur models were used to create the SSM. A

series validation and parametric analysis indicated that the

SSM requires more than 25 distal femur models; each distal

femur should be specified using at least 3000 nodes in space;

and the 3D surface shape prediction should be based on two

2D fluoroscopic images obtained in 450

directions.

Zheng et al. [34] used a 2D/3D reconstruction technique

based on a single image to create a scaled, patient-specific

3D surface model of the pelvis from a single standard AP

X-ray radiograph. This single-image 2D/3D reconstruction

method uses a hybrid 2D/3D deformable registration strategy

that combines landmark-to-ray registration with SSMbased

2D/3D reconstruction.

Ehlke et al. [31] attempted to increase the reliability of

the reconstruction process by considering as much information

as possible about the anatomy of interest. Deformation

of a volumetric tetrahedral mesh with density information

generated by digitally reconstructed radiographic (DRR)

deformations were projected to represent possible candidates

for patient-specific shapes. Compare the X-ray attenuation

of clinical X-rays with the pixel intensity of virtual X-rays to

find the best candidate. This method transfers computations

from the CPU to the GPU, providing interactive frame rates.

Gamage et al. [37] created a 3D reconstruction of the

bone models using salient anatomical edges and contours

computed from orthogonal radiographs. The method

employs an iterative non-rigid 2D point matching methodology

as well as thin-plate spline-based deformation. Noise,

outliers, distortion and occlusions don't affect the non-rigid

registration system. This method was unique in that it

encompasses not only the exterior contours but also several

significant inner edges to ensure adaptability to various bone

anatomies and increase customization accuracy.

Gunay et al. [38] provided a cost and time-effective computational

technique for generating a 3D bone structure

from numerous X-ray images. When projected onto a twodimensional

(2D) plane, this technology scales and deforms

a pre-set 3D template bone structure that is clinically normal

and scaled to an average size until the distorted shape gives

an image equivalent to an input X-ray picture. Sequential

quadratic programming (SQP) was used to achieve multidimensional

optimization by minimizing the error between

the input X-ray image and the image projected from the

deformed template shape.

Mahfouz et al. [39] provided a methodology for reconstructing

lumbar vertebrae from orthogonal views to obtain

precise contour projections. Reconstruction was performed

by deforming the 3D bone model through a robust 3D-2D

registration technique constrained through extracted 2D

morphometric measurements utilizing biplanar X-ray images

and statistical atlas of bone based on Principal Component

Analysis (PCA).

Karade et al. [30] used Laplacian mesh deformation and

self-organizing maps for an effective and precise 3D femur

bone model reconstruction by reforming a 3D template mesh

model to fit bone shape.

By gathering feature information, performing alignment,

and SSM fitting using CNN, Kim et al. [29] have developed

a superior technique for 3D reconstruction of leg bones using

only front X-ray images. The bounding boxes are recognized

by CNN. In situations where boundary outlines are difficult

to extract, feature ellipses and feature points are used

to recognize boundaries, which unifies feature information

detection and saves time and money over manually defining

feature information. It's also more exact and consistent than

the manual assignment.

Koh et al. [33] used three CT images in combination with

two X-ray images and the free form deformation method for

3D reconstruction of patient-specific femurs. It takes very

little time to complete the bone segmentation of three CT

images, the proposed reconstruction approach can be viewed

as being similarly time and cost-effective as the reconstruction

with X-ray images.

Mitton et al. [41] proposed an accurate 3D personalized

model of the pelvis from biplanar X-rays. The method makes

use of both stereo & non- stereo corresponding landmarks,

anatomical atlas (prior knowledge) and contour identification.

The landmarks identified are used to deform the fast

computed initial solution. These retro projected landmarks

on the radiographs are then best matched with the X-rays

contours. Finally, utilizing the corresponding 2D/3D contours

as constraints, an elastic 3D surface deformation

mechanism based on the kriging method is used. The fact

that several anatomical characteristics of the pelvis are not

visible on the two radiographic views from which the reconstruction

is performed is a limitation of the approach.

Three-dimensional reconstruction proved to be very

effective and useful for preoperative planning and is becoming

more important with time. Despite the number of techniques

mentioned above in Table 1 in various literature, the

choice of the method and technology clearly depends upon

the requirement and the region reconstructed.

3 Imaging Requirements of 3D

Reconstruction

Requirements of 3D reconstruction from X-ray images

should be addressed while constructing 3D models, which

are as follows:

i. Acquisition of X-ray image(s): 3D reconstruction from

2D X-ray images can be performed using single X-ray

images [40, 43, 44], two X-ray images [28, 30, 33, 37,

38] or more X-ray images for several anatomical regions

such as femur, tibia, fibula, pelvis and spine. With an

increase in the number of X-ray images, the information

provided for the reconstruction will be increased.

Figure 2 represents a conventional way of X-ray image

acquisition. A portion of the X-ray beam emerging from

the X-ray tube pass through the person’s body where

they are absorbed by the internal structures and the

remaining are transmitted to a detector. It is then use for

further recording or processing by a computer.

ii. Image Enhancement: X-ray images' are low in

intensity, poor in contrast [47]. The quality of the

X-ray image can be improved by applying image

enhancement [48] to improve the assessment. Many

approaches for improving the quality of X-ray images

have been proposed such as histogram equalization

(HE) [49], adaptive histogram equalization (AHE)

[50], wavelet transform coefficients (WT) [51], and

the unsharp masking method (USM) [52, 53]. Huang

et al. [54] employed an adaptive median filter and a

bilateral filter to suppress mixed noise, which includes

both Gaussian and impulsive noise while keeping the

image structures (edges). After that, gray-level morphology

and contrast limiting histogram equalization

(CLAHE) were used to increase the image contrast.

The CLAHE approach enhances fine details, texture,

and local contrast in images. Adaptive contrast

enhancement (ACE) [55] is another well-known local

enhancement method that uses Contrast gains (CG) to

change the high-frequency components of images.

iii. Immobilization during image acquisition: Immobilization

means fixing a body part to reduce or eliminate

the motion of a patient during the image acquisition

process [5]. During image acquisition, it is necessary

to sustain the rigid relationship between the original

anatomy and the acquisition device. Zheng et al. [56]

developed their new immobilization device including

all the anatomical structures.

4 Steps of 3D Reconstruction from 2D X‑ray

Images

There are several types of approaches available for 3D

reconstruction using X-ray images such as based on a

single generic model [17, 41, 57–59], based on statistical

shape and deformation models [60–65]. The former

approaches deform a generic model to build a patientspecific

3D model, but the SSM-based methods build

an SSM to produce the statistically probable models and

decrease the number of parameters to optimize. Hybrid

approaches [66] combine the SSM-based methods with

generic model-based methods. Steps for doing 2D-3D

reconstruction are as follows and are also shown in Fig. 3:

i. 2D X-rays Image acquisition: Input images for 3D

reconstruction derived from X-ray images can be

obtained by acquiring single X-ray images [40, 43,

44], two X-ray images [28, 30, 33, 37, 38] and more

number of X-ray images for femur, tibia, fibula, pelvis

and spine regions. The amount of information will

increase in 3D reconstruction if the number of X-ray

images increases.

ii. Calibration: Due to the non-uniform magnification

of the X-ray images, calibration is necessary for

obtaining linear measurements. Calibration can be

performed for a radiograph using the scale as available

over the radiograph. The scale present on the top

right corner of the craniofacial lateral image helps to

calibrate the lateral image easily. These linear measurements

are important for radiologists, for patients’

diagnosis and treatment planning. The measurement

result can differ if the calibration is not performed correctly

[5].

iii. Contour Extraction (Segmentation): To extract the

edges or contours from the input images, semi-automatic

or manual [40, 58] solutions are available. Different

methods have been offered by various authors

[28, 34, 65, 72, 73] for precise contours extraction

such as Thresholding based, Region-based segmentation,

watershed segmentation, etc. A Canny edge

detector can be applied to detect the edges. The active

Contour method (ACM) provides smooth and closed

contours which are suitable for medical images.

2D/2D correspondences: The process of identifying

correspondences between all points or extracted

contours of two or more X-ray images of the same

scene is known as image correspondences. The point

pair matching or correspondences could be developed

by extracting the points of interest along with some

descriptors and using some (dis)similarity measure

over the descriptors [30]. Pruning the correspondences

by finding the global minimum. Another method could

be finding the iterative distance (Euclidean distance,

city block distance, etc.) between each point to every

point in the other image [28].

v. Registration: Image registration is required in image

analysis applications that include several images of a

scene. It aligns images of a single scene captured from

different angles together to get all the necessary information

without replication. A rigid registration can

be used to register images that are related by rotation,

translation, or scaling. In the registration of medical

images, a 2D/3D correspondence is built between a

3D model (SSM or Mesh model) and 2D X-ray images

to obtain the best match. The features taken from 2D

X-ray images and those recovered from 3D models led

to the 2D/3D registration. In the Non-rigid image registration

category, the images to be registered include

geometric variations. SOM (Self-organizing method)

is one of the non-rigid registration methodology that

Ferrarini et al. [74] used successfully in their GAME

approach.

vi. Shape deformation: The last step is to deform the template

in such a manner that when its silhouette vertices

are projected on a 2D plane, its projection contours

resemble the X-ray image contours. There are differ-

such as Kriging algorithm, Thin-plate splines deformation,

Laplacian and Modified Laplacian [76], Torsional

spring method [77, 78], etc.

5 Computational Approaches for 3D

reconstruction from 2D X‑rays

Models, anatomical knowledge, architectures and applications

were used to classify 3D reconstruction approaches

as shown in Fig. 4. The categorization of 3D image reconstruction

methods are as follows:

5.1 Based on the Information Utilised

for Reconstruction

Intensity-Based Methods: Intensity-based approaches evaluate

the significance of pixel intensities in 2D radiographs

including internal intensity distributions between anatomical

boundaries. To capture the heterogeneity of the region

of interest, volumetric density information is used in the

representation of the 3D anatomical structures. 3D/2D registrations

based on intensity rely entirely on data contained

in voxels and pixels of 3D and 2D images, respectively.

The coinciding points in this method are regarded as corresponding

points and the similarity measure is determined via

pixel-wise comparison instead of the distance. Mutual information,

cross-correlation and a sum of square differences are

the most commonly used similarity measures for DRR-based

3D/2D registrations. One such work of Yao et al. [79] has

developed a deformable density atlas for bone anatomy and

use it in a range of applications, the steps for generating atlas

are as follows:

Construct Tetrahedral Mesh Models from extracted contours

as it is simple, easy to deform and density information

is also stored in the tetrahedron.

• Apply an analytical density function to each tetrahedron

instead of keeping the density value of each pixel in the

model. As a result, computing the measurements such as

integral, differentiating, interpolating and deforming is

simple.

• Once Tetrahedral Mesh Model is built, apply the deformation

technique to it

5.2 Based on the Type of Model Used

Statistical shape modelling: 3D Model built by statistically

analyzing a set of shapes is known as a SSM [32, 60, 63, 80,

81]. The mathematical model contains information on the

shape and its changes. The SSM contains data on the shape

geometry and variances. A Statistical shape model-based

methods are more considered as automated reconstruction

algorithms. These approaches need a large shape learning

database. A Statistical shape modelling system learns from

shape variations of an object or region. The statistical surface

model approach was presented by Cootes et al. [82] and

produced a deformable shape template. The approach integrates

a priori knowledge about the geometrical shape and

its morphological variability. Cootes et al. [82] introduced

the SSM idea as point distribution models (PDMs), which

are deformable representations that convey the mean shape

as well as shape variations. PDMs are created by performing

a PCA on the locations of landmarks that correlate to different

shapes. PCA is used to determine the variations in the

positions of the subject's paired points using a covariance

matrix to create an SSM.

A covariance matrix (Cov) is constructed using the sample

where S is the population, S is a population mean, p is the

number of subjects in the sample set starting from I, (x, y,

z) is the nodes of the sample set in three views and n is the

node number of each model. Then the eigenvalues and the

eigenvector of the (Cov) are computed as shown in Eq. (2).

Using the eigenvalues and eigenvectors, the PCA for subject

models captures geometric features of the models. The

first principal component represents the highest variation

in all models based on direction and location. A new SSM

surface S_ can be obtained via eigen analysis in Eq. (3).

where M is a matrix, S is a population mean, Pj is an eigenvector,

aj is a weight vector used to produce a new shape

model (SSM), and Mi is a transformation matrix used to shift

the ith model in the SSM from its local coordinate system to

the global coordinate system.

Articulated anatomical Models: Articulated statistical

shape models (ASSMs) were first recommended by Heap

et al. [83] as an extended version of the statistical shape

model. It includes a structure of the joint and an analytical

model of a joint to describe the degree of freedom of joint

motion. It allows the model to determine both the shape variation

of individual joint components as well as their usual

range of motion separately. ASSMs are frequently used in

the representation of bones from X-rays. ASSMs convey

relative poses of bones with extra characteristics to represent

diverse postures. An ASSM of the hip joint provided

by Kainmueller et al. [84] used a ball-and-socket joint to

link the relative transformations between the pelvis and the

femur.

5.3 Based on the Method of Model Deformation

Free form deformation: Deformation techniques apply an

adaptable deformation to a template model without taking

into account any statistical data. A geometric approach

called free-form deformation (FFD) [85] is used for basic

deformations of rigid objects. It is built on the concept of

enclosing a template inside a cube (lattice or hull) and altering

it when the cube deforms. The control lattice is designed

to be perpendicular to the model's principal ray. This idea

is called hyper-patches, which are analogues of parametric

curves like Bezier curves [86, 87], B-splines [88, 89], or

NURBs [90, 91]. The template shape deforms until it produces

an image similar to the X-ray image when projected

onto the 2D plane. 3D template model within the lattice

can be modified by changing the control volume parameters

until its projection boundary fits the region contours

retrieved from the X-ray images. The FFD-based technique

demonstrated by Koh et al. [33] used volumetric data in the

form of sparse CT data.

Hierarchical Deformation: The control lattice in hierarchical

deformation [38] is hierarchically segmented in the

regions. The distortion is applied to each region separately

until the template projection resembles the X-ray image

contours. Gunay et al. [38] demonstrated a technique of

Hierarchical FFD. The bounding box is repeatedly split into

smaller boxes, each of which was individually adjusted to get

a form that is closest to the desired shape. Carl Shimer [92]

describes the FFD block as a cubic structure with a hyper

patch on each face. The vectors (S, T, and U) were used to

represent the three sides. The FFD block was defined as an

array of (3l + 1) × (3m + 1) × (3n + 1) hyper patches. This

is essentially a stack of 1 × m × n hyper.

• To complete, create the lattice space which is a local

parametric coordinate system within the FFD block. The

lattice space is defined as follows in Eq. (4):

where X(s, t, u) is the local coordinate system's origin, while

S, T, and U are the edges of the FFD block. Any point inside

the lattice 0 < s < 1, 0 < t < 1and0 < u < 1 is valid. The following

equation Eq. (5) is used to define the control points

on the lattice:

where Pijk is a control points, (S, T and U) are the edges of

the FFD block and (l, m and n) are the integer component.

• After determining the points, the hyper patch deforms by

moving the control points and l × s,m × t, n × u are save

as is, it, andiu to determine the location of the hyper patch

within the FFD block.

• Local coordinate of the system is determined by Eq. (6):

• Calculating position of deformed lattice by putting

back the (u, v,w) values into the hyper patch formula in

Eq. (7):

B(u) is a blending function, Pijk are three-dimensional

control points,

The FFD transformations will apply to all neighboring

regions that tend to deform the object. These approaches

appear to be good alternatives, but they are not adequately

useful in areas where the noise level is high and the useful

data is difficult to distinguish. The FFD approach is

constrained by the fact that it ignores the topology while

deforming it.

5.4 Based on the Architecture Used

Convolutional Neural Network: CNN has lately demonstrated

its effectiveness in a variety of applications, including

image segmentation and classification. CNN for 3D reconstruction

used by Kasten et al. [26] developed a dimensional

expansion method that back-projects each pair of related

epipolar lines into a two-channeled epipolar plane using biplanar

X-rays. This representation was combined with a deep

learning architecture that generates 3D representations of a

bone. The procedure followed by the Kasten et al. [26] was

as follows:

• For training purposes: Each pair of X-ray images was

coupled with aligned ground truth. X-ray images were

created by drawing DRRs from annotated CT scans.

• For controlling the loss function of each training sample;

a spatial 3D Distance Weight Map (DWM) with a size

equal to the ground truth volume and its value on voxel i

was determined by Eq. (8):

where d is a distance transform that describes the distance

between each voxel and each bone surface, and γ = 8, σ = 10

were constants for all the training samples.

• Then, DWM was used for weighting the voxel-wised

cross-entropy loss.

• An unsupervised reconstruction loss was specified to

align the network prediction of the bone’s probability

map with the input X-ray images.

• The overall Loss function is Eq. (9):

CNN as a feature Detection: CNN could be used for automatic

feature analysis [27]. The approach by Kim et al. [29]

extends automation by doing automatic feature analysis of

(7) femur bone using CNN. The structure's position and shape

can be simply determined in the X-ray image as feature elements.

The femur's feature information compensates two

proximal feature points, five distal feature points, and one

ellipse, which is the distinctive shape of the femoral head.

For feature information detection, two CNN-based modules

were used: YOLO (You only look once) v3 [93] and Faster

R-CNN [94] with ROI (Region of Interest) alignment. The

image was fed into these two CNNs, which subsequently

gives a bounding box as an output.

Genetic Algorithm (GA): The GA method is popular

in the field of Artificial Intelligence because of its speedy

search capability and resilience technique [95]. The algorithm

operates in the form of a population, which evolves

several viable solutions over a while (generations) until

it finds the most suitable individual (solution) [96]. Mahfouz

et al. [39] employ a genetic algorithm in conjunction

with their 3D-2D score metric as an optimization technique

to optimize the population, form and alignment of X-ray

images for the reconstruction of lumbar and femur bone. The

procedure used by Mahfouz et al. was as follows:

• Automatic morphometric and surgical axis measurements

were performed on the bone for the registration process

to assess the shape and size of the bone.

• An atlas bone model was aligned with the X-ray images

that provide the initial position.

• The initial population of bone models was created by

combining the initial position with the average model.

• Then, a genetic algorithm was used in conjunction with

a pose score metric as a selection function along with

crossover and mutation. It will improve the population's

structure and alignment with the X-ray images.

• Once the genetic algorithm has converged to a rigid

alignment, the second optimization stage has reached.

• The best member of the population was used as a genetic

dopant.

• After the alignment and registration; affine transformations

(rotation, translation and scaling) are applied for

deformation.

5.5 Hybrid Methods:

This method was derived through the combination of different

approaches. Although these methods have some properties

with the preceding methods, their qualities distinguish

them as a separate class known as hybrid methods. This

class's methods can have a variety of attributes depending

on how they're combined and used. The Kadoury et al. [97]

work is an example of a hybrid approach. He presented a

method for biplanar spine reconstruction that integrated statistical

and image-based methodologies. Their hybrid 3D

reconstruction method merged statistical knowledge with

information derived from images.

5.6 Based on the Anatomical Knowledge Used

for Reconstruction

Point-Based Approaches: Point-based approaches work by

recognizing and matching points on radiographs. Pointbased

methods are divided into SCP (Stereo-Corresponding

Points) and NSCP (Non-Stereo Corresponding Points).

Stereo-Corresponding Point-Based Techniques: SCP

technique is implemented by identifying stereo corresponding

points (SCPs) in radiographs.

(a) The first step is to find the points in two X-ray images

that correlate to each other.

(b) Then, rebuilt in 3D using methods such as Discrete Linear

Transform (DLT).

The DLT approach can be used to find linear mappings

between any two data sets if there are a certain amount of

corresponding data points between them. The number of

SCPs used determines the quality of the results. The more

SCPs used, the better the results [98] but finding more SCP

results is a slow process. Pearcy et al. [99] used SCP and

generate 3D data using the DLT algorithm. Bony formations

with no visible edges are not suited for SCP-based

approaches. Aubin et al. [98] use this method for vertebra

reconstruction utilizing more points (21 SCPs and 6 SCPs

for each vertebra in each radiograph for comparison). The

results were better with 21 SCPs but time-consuming. This

approach is constrained by a number of matching anatomical

landmarks on radiographs.

Non-Stereo Corresponding Point-Based Techniques: To

overcome the limitation of the SCP-based approach, the

NSCP based method (only points visible on a single radiograph

which are having no correspondence with another

image) is introduced as an improvement. This method is

based on the idea that the NSCP is the part of the line connecting

the X-ray source and the point projection in one

view. Steps could be as follows:

(a) The first step in the NSCP method is to calculate the

initial solution. The anatomical area of the general

model is specified.

(b) The 2D contours are then manually identified on the

radiograph. The original 3D model is used to generate

2D contours from each radiograph. The target surface

is then projected on the 2D plane.

(c) The 2D relationship between two given points is based

on the distance between points and contour derivation,

so 2D and 3D contours match. Then the initial response

is optimized in the next step.

d) Finally, the optimal solution is transformed by applying

the Kriging algorithm to it. The depth finding model

used in the kriging algorithm as described by Keaomanee

et al. [100] is depicted in Eq. (10) and Eq. (11),

respectively:

where c0 called nugget and c1 called sill and range a are

kriging parameters determined empirically and _ is a semivariance

used to describe the degree of spatial dependency

between two points (correspondence between 3D model

point and 2D X-ray point).

(e) Finally, the reconstructed shape is obtained by repeating

the last step until the distance between the two

given points is greater than the given accuracy value.

The NSCP-based approach for 3D spine reconstruction

has been proposed by Mitton et al. [101]. Calibration was

performed according to the detected spots. The Shape was

then reconstructed in 3D (using the DLT method).

Parametric Based: Parametric modeling adjusts the shape

of model geometry by changing the dimensional values.

Instead of employing a whole set of points as in SSM, parametric

models examine anatomical descriptive parameters

(DPs) taken from the surface of interest [102]. Depending

on the structure considered, the parametric model consists

of several geometric primitives such as lines, spheres, points

and circles. For example, parametric spine models consist of

points and axes. Humbert et al. [17] suggested a parametric

technique based on transverse and longitudinal inferences

for the 3D reconstruction of lumbar and thoracic radiographs

[103]. The procedure is stated as follows:

• The parameters used are: length (curvature of the spine),

depth, the width of the curve passing through the center

of the vertebral body, the location along the spine curve

of each vertebral endplate.

• These are all then used to build a parametric model of the

spine.

• The parametric model was then back-projected on the

X-ray plane to visualize the 3D geometry of the required

structure.

Contour-Based Methods: Point-based methods cannot be

employed for structures with continuous shapes such as the

nature. Instead of using points, a deformable generic model

with matching contours is used. The non-stereo corresponding

contour (NSCC) method's principle is to link identifiable

2D contours from radiographs to 3D contour. The steps are

as follows:

• Calculate the initial point to initiate contours segmentation

manually or automatically.

• Manually identify 2D outlines on radiographs of the anatomical

area of interest from a generic model.

• On the corresponding radiograph, 3D contours of the

model surface are projected. Then, for each radiograph,

2D contours should be extracted from the 3D initial solution

object.

• After that, an association between these two set points is

conducted. This 2D association is built on point-to-point

distances and contours. This allows for the creation of a

correlation between the 2D and 3D contours.

• The initial solution is then optimized in the next stage.

• The final step is to deform the optimum solution using

the Kriging technique as described by Laporte et al. [57].

• The reconstructed object is achieved by iterating as long

as the distance between two specified points is greater

than a predetermined accuracy value.

For 2D/3D reconstruction, non-statistical geometric

parameters using the contour-based method have been proposed

by Zeng et al. [104] and Karade et al. [30]. Karade

et al. proposed a novel template reconstruction algorithm

that preserves the local properties of the template shape during

transformation. Karade et al. [30] used Laplacian surface

deformation (LSD) to reconstruct 3D structure from two

X-ray images (taken in Mediolateral (ML) and Anteroposterior

(AP) directions). Compared to other deformation algorithms

such as FFD or TPS (Splines in Sheet Metal), LSD

is easier to build and takes less computation time. The input

parameters used in the algorithm were:

• Simulated contours of femur X-ray images derived from

perspective ML (Medio Lateral) and AP (Anterior–Posterior)

views.

• Projections of the 3D model (derived from CT data of the

femur).

• The 3D model of the bone in the form of a 3D mesh with

triangular elements from a clinically normal person. The

model was a 3D point cloud and its projection was also

a 2D image plane point cloud.

As an output, the template model will be rebuilt into a

form that fits the input contours of the 2D image. There

were three steps for reconfiguring the template model. The

first stage involves aligning the template with the input

correlation between input contour points (in both the ML

and AP planes) and silhouette vertices. In the third step,

SOM non-rigid registration was used to find the 2D-2D correspondences

between projection contours of the model and

the input contours to finally have the 2D-3D correspondences.

The steps involved in the process as mentioned by the

Karade et al. [30] are as follows:

“Let ‘K’ be the total number of input contours with pc

k be

the kth input contour (k = 1, 2… K).

Calculate the distance between template projection contours

and input contours as Eq. (12):

where M = total number of template projection points with

(m = 1, 2… M;) and pc

m = mth template projection contour

point and pc

k = kth input contour point.

The best match between template projection contours

and input contours was considered as pp

winner (“nearest to

the input contour”) was selected as Eq. (13):

Each template projection contour point was updated as

Eqs. (14), (15), (16), (17) and (18):

where, tfrac

= currentiterationnumber∕totalnumberofiterations and

and,

SOM output was the adapted template projection contour

points (

pp’) onto the input contour. The 2D–2D correspondence

gives the required 2D–3D correspondence because

the template projection contour points are directly associated

with the silhouette vertices (projection)”. The template

mesh was deformed using LSD as per the estimated 2D–3D

correspondences. The template was deformed such that the

projections of the template's silhouette on both image planes

(ML and AP) have the same shape as the corresponding

input contours. Laplacian surface deformation as a template

deformation approach retains mesh topology and

shape properties. As an output, the deformed mesh with its

silhouette vertices updated to its desired final positions, so

that its projection matches the shape of the input contours.

This approach can be applied to the problem of cavities

that are not clearly seen when viewed perpendicular to the

imaging direction. This approach by Karade et al. [30] may

be used in any area of the skeleton.

6 Evaluation of 3D Reconstruction Methods

of the Literature Surveyed

In terms of the aforementioned features, each recommended

approach in this review paper has some benefits

and drawbacks that should be evaluated before implementation.

Not one method can be chosen as the best for all

applications as every method has its pros and cons which

make them useful as per the situation and need. Table 2

describes various 3D reconstruction methods for different

structures along with their computation time, accuracy

and validation technique used by authors as mentioned

in the literature.

7 Challenges In 2d/3d Reconstruction

Techniques:

Even though the reconstruction of a 3D model from 2D

X-rays images has received a lot of attention in recent

years. Yet, the suggested methods commonly suffer from

one or more of the following practical challenges:

i. Relevant Dataset: Relevant dataset related to the

region of interest is not easily available. To evolve

the 3D reconstruction method, one/multiple X-ray

images along with CT/CBCT is required from the

same patient. Multiple X-ray images and the CT/

CBCT from the same patient are generally not available

in clinics due to the nature of images i.e., radiation

exposure to the patient and overlapping area through

multiple images. To acquire such type of clinical data

for perspective studies is not ethical. Therefore, this

is a major challenge to work in this area.

ii. Feature extraction: Extraction of contours from radiographs

is a complex procedure typically performed

manually [105] or semi-automatically [35, 40]. Manual

segmentation of the anatomy is time-consuming,

takes more effort and requires expertise. As the operator

has to manually outline the contours of the anatomy

on the radiograph. Semi-automatic segmentation

requires an expert to mark the landmarks using the

software.

Large Dataset: When using deep learning algorithms

for 3D reconstruction, a bigger dataset of X-ray

images for training is usually required which is not

easily available [106, 107]. Mostly 3D/2D registration

approach is based on DRRs which are simulated X-ray

projection images because of the difficulty of getting

a real dataset.

8 Discussion

Different authors have employed a variety of strategies for

3D reconstruction of the anatomy of interest utilizing one,

two or more X-ray images. This review paper includes some

of them that are often used for 3D reconstruction. This paper

also includes the steps used for the 3D reconstruction and

challenges encountered during reconstruction. Although

researchers continue to develop new approaches, the core

requirements for 3D reconstruction remain the same for

all the techniques. 3D imaging modalities such as CT and

CBCT rely solely on X-ray images. 3D image reconstruction

from X-ray images can be used to overcome the drawbacks

associated with 3D images like CT and CBCT such as

cost and exposing the patient to higher radiation compared

to the X-ray images. Some reconstruction approaches use

single X-ray images only, while others use X-ray images

along with silhouette-based models, mathematical models

and deformation model techniques. Silhouette-based [108]

reconstruction approaches are easier to use and are more

reliable, that is why many researchers choose to utilize this

approach in their work. 3D template deformation method

uses shape information as a template model, which is then

transformed using the contour information collected from

the calibrated X-ray images. FFD has also emerged as an

option for template deformation for developing 3D structure

reconstruction applications. However, the FFD with few

control points is not effective in preserving the fine features

of the shape. Hierarchical FFD (for accurate shape matching)

is a more complex and time-consuming approach compared

to FFD, but it does not ignore the topology of the

anatomy. Feature-based methods are mostly used compared

to intensity-based methods; the features used are contours,

points and parameters. The point-based procedures are

dependent on the operator's expertise. On multi-view radiographs,

it's difficult to precisely detect and match points. As

a result, point-based approaches do not guarantee repeatability.

Furthermore, due to the limited number of corresponding

anatomical landmarks identifiable on radiographs,

SCP based methods have limited accuracy when used. Pointbased

approaches cannot be employed for structures with

continuous form because of the lack of anatomical landmark

points. The time taken for 3D reconstruction using this procedure

is roughly 2–4 h due to the necessary identification

of points. Thus, these approaches are time-consuming. This

would be a significant drawback that should not be overlooked,

particularly in therapeutic applications and in severe

cases. For a quick and reliable 3D reconstruction, parametric

techniques are ideal. They increased the algorithms' resilience

and convergence. This method results are improved

by repeatability. The intensity-based method [60] is used

to optimize the similarity between real radiographs and

DRR. A volume-based statistical shape and intensity model

(SSIM) developed by Ehlke et al. [31] was used to provide

additional information on the volumetric bone density of

the 3D model. Besides, deep learning methodologies are

also nowadays gaining popularity among researchers. Deep

learning algorithms learn by constructing a more abstract

representation of data, the model extracts feature automatically

and produces greater accuracy outcomes [106]. On a

more precise level, the techniques are separated into deep

learning, silhouette-based, single X-ray based, feature-based,

intensity-based procedures and model deformation-based.

Then there are hybrid approaches. If hybrid strategies are

applied consistently, they can enhance the outcomes [109].

Concerning the above characteristics, each approach recommended

by the author in the literature has some advantages

and disadvantages to consider before implementation. It

is not possible to choose the best one for all applications.

Table 3 shows all these 3D reconstruction methods, each

with its own set of benefits and limitations. It should be

noted that some of the approaches discussed utilized the

EOS system for image acquisition rather than typical radiography

equipment. EOS (Electro-Optical System) device

[110–112] can be an alternative between 2D radiographs and

3D CT scans at lower radiations but it is not readily available

to all clinicians. Its cost is also dependent upon the anatomy

of interest. This has an impact on the accuracy and speed of

reconstruction approaches. If EOS systems are used, more

time may be saved. Photogrammetry for building 3D models

from radiography images has also been suggested as a

potential alternative technique due to its accuracy and low

irradiating dose. Various photogrammetry-based approaches

for 3D image reconstruction from X-ray images have been

proposed and evaluated in [113–115]. DRR images generated

from the perspective projection of a three-dimensional

image onto a two-dimensional plane can be used to create

a relevant dataset of the anatomy of interest, but it does not

guarantee the method's applicability in real scenarios. Less

radiation dosage and low computation cost with little user

supervision are important factors for the acceptability of 3D

reconstruction approaches. 2D/3D reconstruction methods

have the potential to take a place of CT or CBCT imaging in

a non-severe case, if they didn't have access to CT or MRI

equipment. The choice of this reasonably inexpensive and

conveniently accessible method will be advantageous. Furthermore,

a patient's implied risk of cancer can be decreased

In the clinical research, three-dimensional/volumetric anatomical structure of the human body is very significant for diagnosis,

computer-aided surgery, surgical planning, patient follow-up, and biomechanical applications. Medical imaging procedures

including MRI (Magnetic Resonance Imaging), CT (Computed Tomography), and CBCT (Cone-beam computed

tomography) have certain drawbacks such as radiation exposure, availability, and cost. As a result, 3D reconstruction from

2D X-ray images is an alternative way of achieving 3D models with significantly low radiation exposure to the patient. The

purpose of this study is to provide a comprehensive view of 3D image reconstruction methods using X-ray images, and their

applicability in the various anatomical sections of the human body. This study provides a critical analysis of the computational

methods, requirements and steps for 3D reconstruction. This work includes a comparative critical analysis of the

state-of-the-art approaches including the feature selection along with their benefits and drawbacks. This review motivates the

researchers to work for 3D reconstruction using X-ray images as only a limited work is available in the area. It may provide a

solution for many experts who are looking for techniques to reconstruct 3D models from X-ray images for clinical purposes.